Experiment # 5: Vlan

Name : Ahmad Abdullah AL – Qerem .		ID : 11819195 .

Name : Obada Youssef Qasrawi . 			ID : 11822431 .

2st semester 2022/2023 .					Date : 2022/3/15 Thursday .

| Report’s Outcomes |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ILO __ =( ) % | ILO __ =( ) % | ILO __ =( ) % | ILO __ =( ) % | ILO __ =( ) % |  |  |
| Evaluation Criterion |  |  |  |  | Grade | Points |
| Abstract answers of the questions: “What did you do? How did you do it? What did you find?” |  |  |  |  | 0.5 |  |
| Introduction and Theory Sufficient, clear and complete statement of objectives. In addition to Presents sufficiently the theoretical basis. |  |  |  |  | 1.5 |  |
| Apparatus/ Procedure Apparatus sufficiently described to enable another experimenter to identify the equipment needed to conduct the experiment. Procedure sufficiently described. |  |  |  |  | 2 |  |
| Experimental Results and Discussion (In-Lab Worksheet) Crisp explanation of experimental results. Comparison of theoretical predictions to experimental results, including discussion of accuracy and error analysis in some cases. |  |  |  |  | 4 |  |
| Conclusions and Recommendations Conclusions summarize the major findings from the experimental results with adequate specificity. Recommendations appropriate in light of conclusions. Correct grammar. |  |  |  |  | 1 |  |
| Appearance Title page is complete, page numbers applied, content is well organized, correct spelling, fonts are consistent, good visual appeal. |  |  |  |  | 1 |  |
| Total |  |  |  |  | 10 |  |

Abstract :

In this experiment we had learnt how to use switch to create VLANS, using switch

commands , and how to connect two switches with trunk .

Introduction and Theory:

A virtual LAN (VLAN) is a switched network that is logically segmented by function, project team, or application, without regard to the physical locations of the users. Any switch port can belong to a VLAN, and unicast , broadcast, and multicast packets are forwarded and flooded only to stations in the VLAN. Each VLAN is considered a logical network, and packets destined for stations that do not belong to the VLAN must be forwarded through a router or bridge

Objectives :

- Learn how to configure a switch layer2 cisco routers.

- Learn how to configure a switch layer3 cisco routers.

- Learn how to configure VLANs and Trunking.

- Learn how to connect two VLANs using Cisco devices.

Tools used in the experiment:

- Rollover cable.

- 2 PC.

- 2 CISCO Switches .

- Hyper Terminal software.

- 2 Straight patch cords .

- 1 cross-over cord .

Procedure :

Part 1 : ( connect two PC in one VLAN via one switch )

We connected two PCs with the first switch , PC1 ( 192.168.8.101/24 ) , and PC2 ( 192.168.8.102/24 ).

Then we pinged to each other as show below :

Ahmad to obada

obada to Ahmad

We Erased configuration , including VLANs after check VLAN table , with " Sh vlan " as shown below : 						Ahmad

Obada

To delete all VLANs we entered the following command :

Switch#vlan database

Switch (vlan) #no vlan 2

Switch (vlan) #reload

Switch (vlan) exit

Then we changed  switch name to vlan2 via the following commands :

Switch #vlan database

Switch (vlan) vlan 2 name vlan2

Switch (vlan)#exit

Switch #sh vlan brief

- Describe the previous step? Shows all VLANs and their related ports

Switch #sh vlan name vlan2

- Describe the previous step? Shows only the VLAN2 ports

Then we tried to ping to each other again as shown :

Ahmad to Obada

Obada to Ahmad

Both PCs are connected at the same lan via switch .

Then we assigned port f0/2 to vlan2 for PC1 using the following commands :

Switch (conifg)# interface f0/2

Switch (config-if)#switchport mode access

Switch (config-if)#switchport access vlan 2

Switch (config-if)#exit

And the other pc at any port

To see the changes at ports we entered the following command :

Switch #sh vlan brief

Obada

Describe the previous step?

We connected PC1 on switch 1 at port 2 and PC2 on switch 1 at port 1 , here both are not at the same LAN ( deferent VLANs ) then we Ping between PC1 and PC2 as shown :

Ahmad to Obada

Obada to Ahmad

Part 2 : ( connect two PC in one VLAN via two switches )

After we assigned port 2 from both switches for VLAN2  we connected them via trunk at port 20 from both switches , and connected them with crossed cable ( because they are from the same type ) , using the following commands :

Switch # config t

Switch (config)#Interface f0/20

For 3660 switch 				For 2950 switch

Switch (config_if)#Switchport trunk encap dot1q  | Switch (config_if)#Switchport trunk allowed vlan all

Switch (config_if)#Switchport mode trunk | Switch (config_if)#Switchport mode trunk

(Configure the port as a VLAN trunk

for 3660 switch)

Switch (config_if)#exit

Switch #sh vlan brief

Ahmad

Obada

Ping between PC1 and PC2:

Ahmad to Obada

Obada to Ahmad

So both PCs are connected via trunk from switch 1 to switch 2 and both are at the same VLAN , when we ping between them its shows that they are connected each other .

Conclusion:

- We learned about VLANs and how to create them

- We learned about trunk and its fetchers:

- Backup cable .

- Fix storming in switches .

- Higher bandwidth .

- Connect port from the same VLANs at several switches .